Face Frame (3/4 thick)

Stiles 2”

Bottom Rail 2”

Top Rail Base 2”

Top Rail Wall / Tall 3”

Return End Stile Base 3”

Return End Stile Wall 2”

Extended End Stile Base 3 ¾”

Extended End Stile Wall 2 ¾”

To modify Frame sizes:

Click Libraries

Select FACE FRAME

Click the Construction Button

Select the FACE Category

To modify Ret/Ext End Stiles

Click Libraries

Select FACE FRAME

Click the Construction Button

Select the ENDS Category

SCRIBE

Finished End Scribe – 0

Unfinished End Scribe – ½”

To modify Scribe

Click Libraries

Select FACE FRAME

Click the Construction Button

Select the CASE Category

Material Templates

3/4 …

1/2 …

www.mozaiksoftware.com

Method: Face Frame Full Dado

Optimized for flat panel processing

This construction method is for Face Frame construction with full dados.  Bottoms & Tops dado into the ends, partitions dado into the Top & Bottom.  Fixed Shelves dado into the verticals.  Stop dados are used for all parts that are not full depth.

Part Operations.

Case Parts

Dado Depths.

Bottom / Top into End Panels ¼”

Fixed Shelves ¼”

Partitions into Bottom/Top ¼”

Back into End panels ¼”

Adjustable Shelves.

3/16” clearance

Face Frames

Butt Joint construction (Pocket Screws-recommended)

Machining Specifications.

Nested Based Router Setup.

Tool #1	3/8 compression spiral	(Outline tool)

Tool #2	3/8 down shear bit		(dados/pockets)

Tool #3	1/4 down shear bit		(back dados)

Tool #4	1/2 down shear bit		(3/4 dados/pockets)

Drills		5mm

(If no boring block, place a 5mm bit into Tool #5)

Machines with no Tool Changer.

Primary Tool	1/4 compression spiral.

(Adjustable shelf holes will be ¼”.)

Page 2											   www.mozaiksoftware.com